跟骨撬拨夹挤微创复位辅助机器人控制软件 V1.0

源程序鉴别材料

文件：calcaneus_robot\__init__.py

1  """跟骨撬拨夹挤微创复位辅助机器人控制软件。"""

2

3  __version__ = "1.0.0"

文件：calcaneus_robot\controller.py

4  from __future__ import annotations

5

6  from datetime import datetime

7  from typing import Callable

8

9  from .device import SimulatedRobotAdapter

10  from .models import ControlParameters, ControlState, Sample

11  from .storage import RecordStore

12

13

14  class SafetyError(RuntimeError):

15      pass

16

17

18  class RobotController:

19      def __init__(self, device: SimulatedRobotAdapter, store: RecordStore, notify: Callable[[str], None]) -> None:

20          self.device = device

21          self.store = store

22          self.notify = notify

23          self.state = ControlState.DISCONNECTED

24          self.previous_state = ControlState.IDLE

25          self.params = ControlParameters()

26          self.last_sample_second = -1

27

28      def _set_state(self, state: ControlState, message: str) -> None:

29          self.state = state

30          self.store.event("INFO", message)

31          self.notify(message)

32

33      def connect(self) -> None:

34          self.device.connect(); self.device.enable()

35          self._set_state(ControlState.IDLE, "仿真设备连接成功，伺服已使能")

36

37      def disconnect(self) -> None:

38          if self.state not in (ControlState.IDLE, ControlState.COMPLETED, ControlState.DISCONNECTED):

39              raise SafetyError("请先停止当前任务再断开设备")

40          self.device.disconnect()

41          self._set_state(ControlState.DISCONNECTED, "设备已断开")

42

43      def update_parameters(self, params: ControlParameters) -> None:

44          errors = params.validate()

45          if errors:

46              raise ValueError("；".join(errors))

47          if self.state not in (ControlState.IDLE, ControlState.COMPLETED):

48              raise SafetyError("仅可在空闲或完成状态修改参数")

49          self.params = params

50          self.store.event("INFO", "控制参数校验通过")

51

52      def start_positioning(self) -> None:

53          self._require_ready()

54          self.device.progress = 0.0

55          self._set_state(ControlState.POSITIONING, "开始撬拨定位")

56

57      def start_clamping(self) -> None:

58          if self.state not in (ControlState.IDLE, ControlState.COMPLETED):

59              raise SafetyError("当前状态不能启动夹挤复位")

60          self._require_case()

61          self.device.progress = 0.0

62          self._set_state(ControlState.CLAMPING, "开始夹挤复位")

63

64      def pause(self) -> None:

65          if self.state not in (ControlState.POSITIONING, ControlState.CLAMPING, ControlState.HOLDING):

66              raise SafetyError("当前没有可暂停的任务")

67          self.previous_state = self.state

68          self._set_state(ControlState.PAUSED, "任务已暂停")

69

70      def resume(self) -> None:

71          if self.state != ControlState.PAUSED:

72              raise SafetyError("当前任务未暂停")

73          self._set_state(self.previous_state, "任务继续运行")

74

75      def stop(self) -> None:

76          if self.state == ControlState.EMERGENCY:

77              raise SafetyError("急停状态需执行急停复位")

78          if self.state == ControlState.DISCONNECTED:

79              return

80          self._set_state(ControlState.IDLE, "当前任务已安全停止")

81

82      def emergency_stop(self) -> None:

83          self.device.disable()

84          self.store.event("ALARM", "触发软件急停并撤销伺服使能")

85          self.state = ControlState.EMERGENCY

86          self.notify("急停已触发：运动命令被锁定")

87

88      def reset_emergency(self) -> None:

89          if self.state != ControlState.EMERGENCY:

90              raise SafetyError("当前不在急停状态")

91          self.device.reset(); self.device.enable()

92          self._set_state(ControlState.IDLE, "急停复位完成，进入空闲状态")

93

94      def home(self) -> None:

95          if self.state not in (ControlState.IDLE, ControlState.COMPLETED):

96              raise SafetyError("当前状态不能执行软件回零")

97          self.device.reset()

98          self._set_state(ControlState.IDLE, "软件坐标与仿真夹爪已回零")

99

100      def jog_gripper(self, delta_mm: float) -> None:

101          if self.state not in (ControlState.IDLE, ControlState.PAUSED):

102              raise SafetyError("仅可在空闲或暂停状态点动夹爪")

103          self.device.set_gripper(self.device.gripper_opening + delta_mm)

104          self.store.event("INFO", f"夹爪点动至 {self.device.gripper_opening:.1f} mm")

105

106      def tick(self, dt: float = 0.1) -> Sample:

107          pose, wrench, progress, done = self.device.step(self.state, self.params, dt)

108          if wrench.force_norm > self.params.force_limit_n or wrench.torque_norm > self.params.torque_limit_nm:

109              self.store.event("ALARM", f"安全阈值越限：F={wrench.force_norm:.2f}N, T={wrench.torque_norm:.2f}Nm")

110              self.emergency_stop()

111          if done:

112              if self.state in (ControlState.POSITIONING, ControlState.CLAMPING):

113                  self.device.progress = 0.0

114                  self._set_state(ControlState.HOLDING, "目标位移到达，进入保持阶段")

115              elif self.state == ControlState.HOLDING:

116                  self._set_state(ControlState.COMPLETED, "复位辅助流程完成")

117          sample = Sample(datetime.now().isoformat(timespec="milliseconds"), self.state.value, pose, wrench, progress)

118          if self.store.current_case is not None:

119              self.store.sample(sample)

120          return sample

121

122      def _require_ready(self) -> None:

123          if self.state not in (ControlState.IDLE, ControlState.COMPLETED):

124              raise SafetyError("当前状态不能启动撬拨定位")

125          self._require_case()

126

127      def _require_case(self) -> None:

128          if self.store.current_case is None:

129              raise SafetyError("请先建立病例")

130          if not self.device.connected or not self.device.servo_enabled:

131              raise SafetyError("设备未连接或伺服未使能")

文件：calcaneus_robot\device.py

132  from __future__ import annotations

133

134  import random

135  from dataclasses import replace

136

137  from .models import ControlParameters, ControlState, ForceTorque, Pose

138

139

140  class DeviceError(RuntimeError):

141      pass

142

143

144  class SimulatedRobotAdapter:

145      """离线仿真适配器；不连接或驱动任何真实医疗设备。"""

146

147      def __init__(self) -> None:

148          self.connected = False

149          self.servo_enabled = False

150          self.pose = Pose()

151          self.wrench = ForceTorque()

152          self.progress = 0.0

153          self.gripper_opening = 30.0

154          self._hold_elapsed = 0.0

155

156      def connect(self) -> None:

157          self.connected = True

158

159      def disconnect(self) -> None:

160          self.servo_enabled = False

161          self.connected = False

162

163      def enable(self) -> None:

164          if not self.connected:

165              raise DeviceError("设备尚未连接")

166          self.servo_enabled = True

167

168      def disable(self) -> None:

169          self.servo_enabled = False

170

171      def reset(self) -> None:

172          if not self.connected:

173              raise DeviceError("设备尚未连接")

174          self.pose = Pose()

175          self.wrench = ForceTorque()

176          self.progress = 0.0

177          self.gripper_opening = 30.0

178          self._hold_elapsed = 0.0

179

180      def set_gripper(self, opening_mm: float) -> None:

181          if not self.connected or not self.servo_enabled:

182              raise DeviceError("设备未连接或伺服未使能")

183          self.gripper_opening = max(0.0, min(40.0, opening_mm))

184

185      def step(self, state: ControlState, p: ControlParameters, dt: float) -> tuple[Pose, ForceTorque, float, bool]:

186          if not self.connected:

187              return self.pose, self.wrench, self.progress, False

188          noise = lambda scale: random.uniform(-scale, scale)

189          done = False

190          if state == ControlState.POSITIONING:

191              self.pose.z = min(p.target_pry_mm, self.pose.z + p.speed_mm_s * dt)

192              self.progress = 100.0 if p.target_pry_mm == 0 else self.pose.z / p.target_pry_mm * 100.0

193              self.wrench = ForceTorque(noise(0.8), 5 + self.pose.z * 1.7 + noise(1), 9 + self.pose.z * 2.4 + noise(1.2), noise(.15), .4 + noise(.1), noise(.1))

194              done = self.pose.z >= p.target_pry_mm

195          elif state == ControlState.CLAMPING:

196              traveled = 30.0 - self.gripper_opening

197              traveled = min(p.target_clamp_mm, traveled + p.speed_mm_s * dt)

198              self.gripper_opening = 30.0 - traveled

199              self.pose.x = traveled

200              self.progress = 100.0 if p.target_clamp_mm == 0 else traveled / p.target_clamp_mm * 100.0

201              self.wrench = ForceTorque(8 + traveled * 2.8 + noise(1.4), noise(.8), 12 + traveled * 1.4 + noise(1), .3 + noise(.1), noise(.12), .6 + traveled * .12 + noise(.1))

202              done = traveled >= p.target_clamp_mm

203          elif state == ControlState.HOLDING:

204              self._hold_elapsed += dt

205              self.progress = min(100.0, self._hold_elapsed / p.hold_seconds * 100.0)

206              self.wrench = replace(self.wrench, fx=self.wrench.fx + noise(.5), fz=self.wrench.fz + noise(.5))

207              done = self._hold_elapsed >= p.hold_seconds

208          elif state in (ControlState.IDLE, ControlState.PAUSED, ControlState.COMPLETED):

209              self.wrench = ForceTorque(*(v * .92 for v in (self.wrench.fx, self.wrench.fy, self.wrench.fz, self.wrench.tx, self.wrench.ty, self.wrench.tz)))

210          return self.pose, self.wrench, min(100.0, self.progress), done

文件：calcaneus_robot\models.py

211  from __future__ import annotations

212

213  from dataclasses import asdict, dataclass, field

214  from datetime import datetime

215  from enum import Enum

216  from math import sqrt

217  from typing import Any

218

219

220  class ControlState(str, Enum):

221      DISCONNECTED = "设备未连接"

222      IDLE = "空闲"

223      POSITIONING = "撬拨定位"

224      CLAMPING = "夹挤复位"

225      HOLDING = "保持"

226      PAUSED = "已暂停"

227      EMERGENCY = "急停锁定"

228      COMPLETED = "流程完成"

229

230

231  @dataclass

232  class ForceTorque:

233      fx: float = 0.0

234      fy: float = 0.0

235      fz: float = 0.0

236      tx: float = 0.0

237      ty: float = 0.0

238      tz: float = 0.0

239

240      @property

241      def force_norm(self) -> float:

242          return sqrt(self.fx**2 + self.fy**2 + self.fz**2)

243

244      @property

245      def torque_norm(self) -> float:

246          return sqrt(self.tx**2 + self.ty**2 + self.tz**2)

247

248

249  @dataclass

250  class Pose:

251      x: float = 0.0

252      y: float = 0.0

253      z: float = 0.0

254      rx: float = 0.0

255      ry: float = 0.0

256      rz: float = 0.0

257

258

259  @dataclass

260  class PatientCase:

261      case_id: str

262      patient_code: str

263      side: str

264      operator: str

265      note: str = ""

266      created_at: str = field(default_factory=lambda: datetime.now().isoformat(timespec="seconds"))

267

268      def to_dict(self) -> dict[str, Any]:

269          return asdict(self)

270

271

272  @dataclass

273  class ControlParameters:

274      target_pry_mm: float = 8.0

275      target_clamp_mm: float = 12.0

276      speed_mm_s: float = 1.0

277      force_limit_n: float = 80.0

278      torque_limit_nm: float = 8.0

279      hold_seconds: float = 3.0

280

281      def validate(self) -> list[str]:

282          errors: list[str] = []

283          if not 0.0 <= self.target_pry_mm <= 30.0:

284              errors.append("撬拨位移应为 0～30 mm")

285          if not 0.0 <= self.target_clamp_mm <= 40.0:

286              errors.append("夹挤位移应为 0～40 mm")

287          if not 0.1 <= self.speed_mm_s <= 5.0:

288              errors.append("运动速度应为 0.1～5.0 mm/s")

289          if not 5.0 <= self.force_limit_n <= 150.0:

290              errors.append("力上限应为 5～150 N")

291          if not 0.5 <= self.torque_limit_nm <= 15.0:

292              errors.append("力矩上限应为 0.5～15 Nm")

293          if not 1.0 <= self.hold_seconds <= 30.0:

294              errors.append("保持时间应为 1～30 s")

295          return errors

296

297

298  @dataclass

299  class Sample:

300      timestamp: str

301      state: str

302      pose: Pose

303      wrench: ForceTorque

304      progress: float

305

306      def to_row(self) -> list[str]:

307          return [

308              self.timestamp, self.state,

309              f"{self.pose.x:.3f}", f"{self.pose.y:.3f}", f"{self.pose.z:.3f}",

310              f"{self.pose.rx:.3f}", f"{self.pose.ry:.3f}", f"{self.pose.rz:.3f}",

311              f"{self.wrench.fx:.3f}", f"{self.wrench.fy:.3f}", f"{self.wrench.fz:.3f}",

312              f"{self.wrench.tx:.3f}", f"{self.wrench.ty:.3f}", f"{self.wrench.tz:.3f}",

313              f"{self.wrench.force_norm:.3f}", f"{self.progress:.1f}",

314          ]

文件：calcaneus_robot\storage.py

315  from __future__ import annotations

316

317  import csv

318  import json

319  from datetime import datetime

320  from pathlib import Path

321

322  from .models import PatientCase, Sample

323

324

325  class RecordStore:

326      HEADER = ["时间", "状态", "X(mm)", "Y(mm)", "Z(mm)", "Rx(deg)", "Ry(deg)", "Rz(deg)", "Fx(N)", "Fy(N)", "Fz(N)", "Tx(Nm)", "Ty(Nm)", "Tz(Nm)", "合力(N)", "进度(%)"]

327

328      def __init__(self, root: Path) -> None:

329          self.root = root

330          self.root.mkdir(parents=True, exist_ok=True)

331          self.current_case: PatientCase | None = None

332          self.samples: list[Sample] = []

333          self.events: list[dict[str, str]] = []

334

335      def begin_case(self, case: PatientCase) -> None:

336          self.current_case = case

337          self.samples.clear()

338          self.events.clear()

339          self.event("INFO", f"新建病例：{case.case_id}")

340

341      def event(self, level: str, message: str) -> None:

342          self.events.append({"time": datetime.now().isoformat(timespec="seconds"), "level": level, "message": message})

343

344      def sample(self, value: Sample) -> None:

345          self.samples.append(value)

346

347      def export(self) -> Path:

348          if self.current_case is None:

349              raise ValueError("尚未建立病例")

350          folder = self.root / self.current_case.case_id

351          folder.mkdir(parents=True, exist_ok=True)

352          (folder / "case.json").write_text(json.dumps(self.current_case.to_dict(), ensure_ascii=False, indent=2), encoding="utf-8")

353          (folder / "events.json").write_text(json.dumps(self.events, ensure_ascii=False, indent=2), encoding="utf-8")

354          with (folder / "samples.csv").open("w", encoding="utf-8-sig", newline="") as f:

355              writer = csv.writer(f); writer.writerow(self.HEADER); writer.writerows(s.to_row() for s in self.samples)

356          report = folder / "复位过程报告.txt"

357          max_force = max((s.wrench.force_norm for s in self.samples), default=0.0)

358          report.write_text(

359              f"跟骨微创复位辅助机器人过程报告\n病例编号：{self.current_case.case_id}\n患者编码：{self.current_case.patient_code}\n患侧：{self.current_case.side}\n操作者：{self.current_case.operator}\n采样数：{len(self.samples)}\n最大合力：{max_force:.2f} N\n生成时间：{datetime.now():%Y-%m-%d %H:%M:%S}\n声明：本报告由离线演示原型生成，不构成医疗诊断或临床记录。\n",

360              encoding="utf-8",

361          )

362          return folder

文件：calcaneus_robot\ui.py

363  from __future__ import annotations

364

365  import tkinter as tk

366  from datetime import datetime

367  from pathlib import Path

368  from tkinter import messagebox, ttk

369

370  from .controller import RobotController

371  from .device import SimulatedRobotAdapter

372  from .models import ControlParameters, ControlState, PatientCase

373  from .storage import RecordStore

374

375

376  class CalcaneusRobotApp(tk.Tk):

377      def __init__(self) -> None:

378          super().__init__()

379          self.withdraw()

380          self.title("跟骨撬拨夹挤微创复位辅助机器人控制软件 V1.0")

381          self.geometry("1280x780"); self.minsize(1100, 700)

382          self.option_add("*Font", ("Microsoft YaHei UI", 10))

383          root = Path(__file__).resolve().parents[1]

384          self.store = RecordStore(root / "data")

385          self.controller = RobotController(SimulatedRobotAdapter(), self.store, self.log)

386          self.vars = {k: tk.StringVar(value="0.00") for k in ("fx","fy","fz","tx","ty","tz","force","x","y","z","opening")}

387          self.state_var = tk.StringVar(value=ControlState.DISCONNECTED.value)

388          self.progress_var = tk.DoubleVar(value=0)

389          if self.login_dialog():

390              self._build(); self.deiconify(); self.after(100, self.refresh)

391          else:

392              self.destroy()

393

394      def login_dialog(self) -> bool:

395          dialog = tk.Toplevel(self); dialog.title("用户登录"); dialog.geometry("360x210"); dialog.resizable(False, False); dialog.grab_set()

396          ok = {"value": False}; user=tk.StringVar(value="operator"); pwd=tk.StringVar(value="demo123")

397          ttk.Label(dialog,text="操作员账号").pack(pady=(22,4)); ttk.Entry(dialog,textvariable=user).pack(fill="x",padx=55)

398          ttk.Label(dialog,text="密码").pack(pady=(9,4)); ttk.Entry(dialog,textvariable=pwd,show="*").pack(fill="x",padx=55)

399          def submit():

400              if user.get()=="operator" and pwd.get()=="demo123": ok["value"]=True; dialog.destroy()

401              else: messagebox.showerror("登录失败","账号或密码错误",parent=dialog)

402          ttk.Button(dialog,text="登录",command=submit).pack(pady=15); dialog.protocol("WM_DELETE_WINDOW",dialog.destroy)

403          self.wait_window(dialog); return ok["value"]

404

405      def _build(self) -> None:

406          style=ttk.Style(); style.configure("Alarm.TButton",foreground="#b00020",font=("Microsoft YaHei UI",11,"bold"))

407          top=ttk.Frame(self,padding=8); top.pack(fill="x")

408          ttk.Label(top,text="跟骨撬拨夹挤微创复位辅助机器人控制软件",font=("Microsoft YaHei UI",16,"bold")).pack(side="left")

409          ttk.Label(top,textvariable=self.state_var,foreground="#075a9c",font=("Microsoft YaHei UI",12,"bold")).pack(side="right",padx=12)

410          ttk.Button(top,text="连接仿真设备",command=lambda:self.safe(self.controller.connect)).pack(side="right",padx=3)

411          ttk.Button(top,text="断开",command=lambda:self.safe(self.controller.disconnect)).pack(side="right",padx=3)

412          body=ttk.Panedwindow(self,orient="horizontal"); body.pack(fill="both",expand=True,padx=8,pady=(0,8))

413          left=ttk.Frame(body); center=ttk.Frame(body); right=ttk.Frame(body); body.add(left,weight=2); body.add(center,weight=3); body.add(right,weight=2)

414          self._case_panel(left); self._parameter_panel(left); self._control_panel(left)

415          self._visual_panel(center); self._wrench_panel(center)

416          self._status_panel(right); self._log_panel(right)

417

418      def group(self,parent,title):

419          f=ttk.LabelFrame(parent,text=title,padding=8); f.pack(fill="x",padx=4,pady=4); return f

420

421      def _case_panel(self,parent):

422          f=self.group(parent,"病例与任务"); self.case_entries={}

423          defaults={"病例编号":f"CASE-{datetime.now():%Y%m%d-%H%M}","患者编码":"P001","操作者":"operator"}

424          for i,(name,val) in enumerate(defaults.items()):

425              ttk.Label(f,text=name).grid(row=i,column=0,sticky="w",pady=2); v=tk.StringVar(value=val); ttk.Entry(f,textvariable=v,width=22).grid(row=i,column=1,sticky="ew"); self.case_entries[name]=v

426          ttk.Label(f,text="患侧").grid(row=3,column=0,sticky="w"); self.side=tk.StringVar(value="左"); ttk.Combobox(f,textvariable=self.side,values=("左","右"),state="readonly",width=19).grid(row=3,column=1)

427          ttk.Button(f,text="建立病例",command=self.create_case).grid(row=4,column=0,columnspan=2,sticky="ew",pady=(7,0)); f.columnconfigure(1,weight=1)

428

429      def _parameter_panel(self,parent):

430          f=self.group(parent,"规划与安全参数"); self.param_entries={}

431          items=(("撬拨位移(mm)",8.0),("夹挤位移(mm)",12.0),("速度(mm/s)",1.0),("力上限(N)",80.0),("力矩上限(Nm)",8.0),("保持时间(s)",3.0))

432          for i,(name,val) in enumerate(items):

433              ttk.Label(f,text=name).grid(row=i,column=0,sticky="w",pady=2); v=tk.StringVar(value=str(val)); ttk.Entry(f,textvariable=v,width=12).grid(row=i,column=1); self.param_entries[name]=v

434          ttk.Button(f,text="校验并应用参数",command=self.apply_params).grid(row=len(items),column=0,columnspan=2,sticky="ew",pady=(7,0))

435

436      def _control_panel(self,parent):

437          f=self.group(parent,"复位辅助控制")

438          buttons=(("开始撬拨定位",self.controller.start_positioning),("开始夹挤复位",self.controller.start_clamping),("暂停",self.controller.pause),("继续",self.controller.resume),("安全停止",self.controller.stop),("软件回零",self.controller.home),("夹爪张开 +1mm",lambda:self.controller.jog_gripper(1)),("夹爪闭合 -1mm",lambda:self.controller.jog_gripper(-1)))

439          for i,(text,cmd) in enumerate(buttons): ttk.Button(f,text=text,command=lambda c=cmd:self.safe(c)).grid(row=i//2,column=i%2,sticky="ew",padx=2,pady=2)

440          ttk.Button(f,text="急停",style="Alarm.TButton",command=self.controller.emergency_stop).grid(row=4,column=0,sticky="ew",padx=2,pady=5)

441          ttk.Button(f,text="急停复位",command=lambda:self.safe(self.controller.reset_emergency)).grid(row=4,column=1,sticky="ew",padx=2,pady=5)

442          for i in range(2): f.columnconfigure(i,weight=1)

443

444      def _visual_panel(self,parent):

445          f=ttk.LabelFrame(parent,text="二维术野与机器人运动示意",padding=5); f.pack(fill="both",expand=True,padx=4,pady=4)

446          self.canvas=tk.Canvas(f,bg="#eef3f6",height=360,highlightthickness=0); self.canvas.pack(fill="both",expand=True)

447          self.canvas.bind("<Configure>",lambda e:self.draw_scene())

448

449      def _wrench_panel(self,parent):

450          f=self.group(parent,"六维力/力矩实时监测")

451          for i,key in enumerate(("fx","fy","fz","tx","ty","tz","force")):

452              label={"fx":"Fx (N)","fy":"Fy (N)","fz":"Fz (N)","tx":"Tx (Nm)","ty":"Ty (Nm)","tz":"Tz (Nm)","force":"合力 (N)"}[key]

453              box=ttk.Frame(f); box.grid(row=i//4,column=i%4,padx=8,pady=4); ttk.Label(box,text=label).pack(); ttk.Label(box,textvariable=self.vars[key],font=("Consolas",13,"bold")).pack()

454

455      def _status_panel(self,parent):

456          f=self.group(parent,"位置与流程状态")

457          for i,key in enumerate(("x","y","z","opening")):

458              ttk.Label(f,text={"x":"夹挤行程 X","y":"横向位置 Y","z":"撬拨高度 Z","opening":"夹爪开度"}[key]).grid(row=i,column=0,sticky="w",pady=3)

459              ttk.Label(f,textvariable=self.vars[key],font=("Consolas",12,"bold")).grid(row=i,column=1,sticky="e")

460          self.progress=ttk.Progressbar(f,variable=self.progress_var,maximum=100); self.progress.grid(row=4,column=0,columnspan=2,sticky="ew",pady=8)

461          ttk.Button(f,text="导出过程记录",command=self.export).grid(row=5,column=0,columnspan=2,sticky="ew"); f.columnconfigure(1,weight=1)

462

463      def _log_panel(self,parent):

464          f=ttk.LabelFrame(parent,text="事件日志",padding=5); f.pack(fill="both",expand=True,padx=4,pady=4)

465          self.log_text=tk.Text(f,height=18,state="disabled",wrap="word",font=("Consolas",9)); self.log_text.pack(fill="both",expand=True)

466

467      def create_case(self):

468          try:

469              values={k:v.get().strip() for k,v in self.case_entries.items()}

470              if not all(values.values()): raise ValueError("病例编号、患者编码和操作者不能为空")

471              self.store.begin_case(PatientCase(values["病例编号"],values["患者编码"],self.side.get(),values["操作者"]))

472              self.log("病例建立成功")

473          except Exception as e: messagebox.showerror("建立失败",str(e),parent=self)

474

475      def apply_params(self):

476          try:

477              v={k:float(x.get()) for k,x in self.param_entries.items()}

478              p=ControlParameters(v["撬拨位移(mm)"],v["夹挤位移(mm)"],v["速度(mm/s)"],v["力上限(N)"],v["力矩上限(Nm)"],v["保持时间(s)"])

479              self.controller.update_parameters(p); self.log("参数已应用")

480          except Exception as e: messagebox.showerror("参数错误",str(e),parent=self)

481

482      def safe(self,func):

483          try: func()

484          except Exception as e: self.log(f"操作拒绝：{e}"); messagebox.showwarning("操作未执行",str(e),parent=self)

485

486      def refresh(self):

487          try:

488              s=self.controller.tick(.1); w=s.wrench; p=s.pose

489              for k,val in (("fx",w.fx),("fy",w.fy),("fz",w.fz),("tx",w.tx),("ty",w.ty),("tz",w.tz),("force",w.force_norm),("x",p.x),("y",p.y),("z",p.z),("opening",self.controller.device.gripper_opening)): self.vars[k].set(f"{val:.2f}")

490              self.state_var.set(self.controller.state.value); self.progress_var.set(s.progress); self.draw_scene()

491          finally: self.after(100,self.refresh)

492

493      def draw_scene(self):

494          if not hasattr(self,"canvas"): return

495          c=self.canvas; c.delete("all"); w=max(c.winfo_width(),500); h=max(c.winfo_height(),300)

496          c.create_text(15,15,anchor="nw",text="离线仿真视图（非医学影像）",fill="#6b7680")

497          c.create_oval(w*.32,h*.35,w*.7,h*.72,fill="#e6c3a5",outline="#9c6d50",width=3)

498          c.create_polygon(w*.38,h*.52,w*.48,h*.37,w*.62,h*.49,w*.57,h*.66,w*.42,h*.67,fill="#f7eadc",outline="#8b6d57",width=2)

499          z=float(self.vars["z"].get()); x=float(self.vars["x"].get())

500          c.create_line(w*.5,h*.14,w*.5,h*(.43-z/120),fill="#286a9b",width=8,arrow="last")

501          gap=max(25,75-x*2); c.create_line(w*.29,h*.55,w*.5-gap,h*.55,fill="#3c4650",width=10); c.create_line(w*.71,h*.55,w*.5+gap,h*.55,fill="#3c4650",width=10)

502          c.create_text(w*.5,h*.84,text=f"撬拨 {z:.1f} mm    夹挤 {x:.1f} mm",font=("Microsoft YaHei UI",12,"bold"),fill="#163c5a")

503

504      def log(self,message):

505          if not hasattr(self,"log_text"): return

506          self.log_text.configure(state="normal"); self.log_text.insert("end",f"[{datetime.now():%H:%M:%S}] {message}\n"); self.log_text.see("end"); self.log_text.configure(state="disabled")

507

508      def export(self):

509          try: folder=self.store.export(); self.log(f"记录已导出：{folder}"); messagebox.showinfo("导出完成",f"文件已保存至：\n{folder}",parent=self)

510          except Exception as e: messagebox.showerror("导出失败",str(e),parent=self)

文件：main.py

511  from calcaneus_robot.ui import CalcaneusRobotApp

512

513

514  def main() -> None:

515      app = CalcaneusRobotApp()

516      app.mainloop()

517

518

519  if __name__ == "__main__":

520      main()